*****************************AUTOEXEC.NET**************************************

#

# General setup.  Note that all IP addresses and domain names are fictitious.

#

scrollback 0

isat on

hostname myhost.domain.name

#

# Set up ethernet link

#

attach packet 0x60 en0 20 1500

ifconfig en0 ipaddr 192.0.2.1

ifconfig en0 netmask 0xffffff00

ifconfig en0 broadcast 192.0.2.255

arp add 192.0.2.255 ethernet ff:ff:ff:ff:ff:ff

#

# Set up autodial Van Jacobsen compressed SLIP link @ 57.6K bps

# with RTS-CTS flow control.

#

#attach asy 0x3f8 4 vjslip sl0 4096 1500 57600 cr

#ifconfig sl0 ipaddr 1.0.0.1

#ifconfig sl0 netmask 0xffffff00

#ifconfig sl0 broadcast 1.0.0.255

#

# Broadcast default route with RIP every 30 seconds

#

route add default sl0 1.0.0.2 1

start rip

rip add 192.0.2.255 30 1

#

# Configure name lookup

#

domain suffix domain.name

domain addserver 192.0.2.2

#

# Start daemons.

#

start ftp

remote -s ratinthevat

start remote

#

# Start packet logging.

#

ip log 192.0.2.2:514

#

# Set up packet filter on SLIP link.

#

ip filter sl0 permit in tcpxsyn !192.0.2.0/24     192.0.2.0/24

ip filter sl0 permit in icmpxrd !192.0.2.0/24     192.0.2.0/24

ip filter sl0 permit in udp     !192.0.2.0/24:53  192.0.2.0/24:53

ip filter sl0 permit in udp     !192.0.2.0/24:53  192.0.2.0/24:1024+

ip filter sl0 permit in udp     !192.0.2.0/24:123 192.0.2.0/24:123

ip filter sl0 permit in tcpsyn  !192.0.2.0/24:20  192.0.2.0/24:1024+

ip filter sl0 permit in tcpsyn  !192.0.2.0/24     192.0.2.0/24:25

ip filter sl0 permit in tcpsyn  !192.0.2.0/24     192.0.2.2:79

ip filter sl0 log deny in *     *                 *

ip filter sl0 permit out *      192.0.2.0/24      !192.0.2.0/24

#

# Enable dialer.  Do this last so filtering will be up before the link.

#

dialer sl0 1000 SLIPDIAL SLIPDOWN SLIPDOWN

*******************************************************************************

AUTOEXEC.NET is the KA9Q initialization file read at startup.

*********************************SLIPDIAL**************************************

speed 57600

control up

wait 500

send "ATDTW 555-1212\r"

wait 35000 "CONNECT"

wait 20000 "login: "

send "user69\r"

wait 5000 "Password: "

send "password105\r"

wait 5000 "Packet mode enabled"

*******************************************************************************

SLIPDIAL is the autodialer script executed when the link is down and

packets need to be routed.

*********************************SLIPDOWN**************************************

control down

wait 5000

control up

*******************************************************************************

SLIPDOWN is the script executed when the link has been idle for the

interval specified in the dialer command.

*********************************FTPUSERS**************************************

mischler siegelgrund / 127

*******************************************************************************