Scancode Conversion Tables.

Int 09h translates scancodes to ASCII character codes or other

hexadecimal codes as shown in the tables below. The first table

shows scancodes returned when the Char function is OFF.  The

second table shows scancodes returned when the Char function is ON.

For each scancode, the

tables give the equivalent character codes for each keyboard state:

normal, shifted, [Ctrl] active, and [Alt] active.  If a scancode

has an ASCII equivalent, the ASCII character is returned in register AL.

If a converted code of the form

"xxh/00" is shown, a zero value is returned in register AL to indicate

that there is no ASCII value for the key combination.  BIOS Int 16h

returns the value "XXh" in register AH.

Note:

Some shifted characters on Jaguar are non-shifted on standard IBM keyboards

and some non-shifted characters on Jaguar are shifted on standard

IBM keyboards.  For example,

[(] is non-shifted on Jaguar and shifted on IBM

[Home] is shifted on Jaguar and non-shifted on IBM.

On these non-compatible key mappings, Int 0Bh will force

the status of the shift bits in 40h:17h

to the state used by a compatible keyboard, regardless of

the actual state of the shift keys.  For example:

[(] is reported to Int 09h as a shifted character even though it

is non-shifted on Jaguar.

[Home] is reported to Int 09h as a non-shifted character even though

it is shifted on Jaguar.

In IBM compatible machines, the [Shift], [Ctrl], and [Alt] keys

affect the interpretation of a scancode with ascending priority.  That

is, the [Alt] key has the highest priority.  If [Alt] is pressed, the

[Shift] and [Ctrl] keys have no affect.  Likewise, if [Ctrl] is pressed,

the [Shift] key has no affect.  The only valid combination involving

both [Ctrl] and [Alt] is the Warm Start sequence [CTRL][ALT][DEL].

In Jaguar, however, there are some valid combinations of [Shift] and [Ctrl].

In particular:

[Shift][Ctrl][PGUP] yields the compatible keycode for [Ctrl][PGUP]

[Shift][Ctrl][HOME] yields the compatible keycode for [Ctrl][HOME]

[Shift][Ctrl][PGDN] yields the compatible keycode for [Ctrl][PGDN]

[Shift][Ctrl][END] yields the compatible keycode for [Ctrl][END]

[Shift][Ctrl][ [ ] yields the compatible keycode for [Ctrl][ [ ]

[Shift][Ctrl][ ] ] yields the compatible keycode for [Ctrl][ ] ]

******************************************************************************

SCANCODE TABLE WHEN CHAR FUNCTION IS OFF

CHARACTER CODES (hex)

KEY LEGEND	SCANCODE	Normal	Shifted	Control	Alt	Shift+Control

Normal	Shifted	Normal	Shifted	ah/al	ah/al	ah/al	ah/al	ah/al

ESC	PrtScr	01	37	01/1B	PrtScr	01/1B

TAB 		0F 	0F 	0F/09 	0F/00	94/00	A5/00	94/00

F1		3B	3B	3B/00	54/00	5E/00	68/00	5E/00

F2		3C	3C	3C/00	55/00	5F/00	69/00	5F/00

F3		3D	3D	3D/00	56/00	60/00	6A/00	60/00

F4		3E	3E	3E/00	57/00	61/00	6B/00	61/00

F5		3F	3F	3F/00	58/00	62/00	6C/00	62/00

F6		40	40	40/00	59/00	63/00	6D/00	63/00

F7		41	41	41/00	5A/00	64/00	6E/00	64/00

F8		42	42	42/00	5B/00	65/00	6F/00	65/00

F9		43	43	43/00	5C/00	66/00	70/00	66/00

F10		44	44	44/00	5D/00	67/00	71/00	67/00

upcurs	PGUP	48	49	48/00	49/00	8D/00		84/00

ON/OFF 	ON/OFF	70	70			A2/00

FILER	SETUP	72	71	A8/00	A4/00	AE/00	AB/00	AA/00

COMM	`	73	29	AC/00	29/60	B2/00	AF/00

APPT	~	74	29	B0/00	29/7E	B6/00	B3/00

PHONE	!	75	02	B4/00	02/21	BA/00	B7/00

MEMO	#	76	04	B8/00	04/23	BE/00	BB/00

123	$	77	05	BC/00	05/24	C2/00	BF/00

+-*/	&	78	08	C0/00	08/26	C6/00	C3/00

(	|	0A	2B	0A/28	2B/7C		80/00	2B/1C

)	\	0B	2B	0B/29	2B/5C		81/00	2B/1C

<--	<--	0E	0E	0E/08	0E/08	BREAK	0E/00	BREAK

DEL	INS	53	52	53/00	52/00	93/00		92/00

lfcurs	HOME	4B	47	4B/00	47/00	73/00		77/00

dncurs	PGDN	50	51	50/00	51/00	91/00		76/00

rtcurs	END	4D	4F	4D/00	4F/00	74/00		75/00

Q 		10	10	10/71	10/51	10/11	10/00	10/11

W 		11	11	11/77	11/57	11/17	11/00	11/17

E 		12	12	12/65	12/45	12/05	12/00	12/05

R 		13	13	13/72	13/52	13/12	13/00	13/12

T 		14	14	14/74	14/54	14/14	14/00	14/14

Y 		15	15	15/79	15/59	15/19	15/00	15/19

U		16	16	16/75	16/55	16/15	16/00	16/15

I		17	17	17/69	17/49	17/09	17/00	17/09

O		18	18	18/6F	18/4F	18/0F	18/00	18/0F

P		19	19	19/70	19/50	19/10	19/00	19/10

7	[	08	1A	08/37	1A/5B			1A/1B

8	]	09	1B	09/38	1B/5D			1B/1D

9	{	0A	1A	0A/39	1A/7B			1A/1B

/	}	35	1B	35/2F	1B/7D			1B/1D

SCANCODE TABLE WHEN CHAR FUNCTION IS OFF

CHARACTER CODES (hex)

KEY LEGEND	SCANCODE	Normal	Shifted	Control	Alt	Shift+Control

Normal	Shifted	Normal	Shifted	ah/al	ah/al	ah/al	ah/al	ah/al

A		1E	1E	1E/61	1E/41	1E/01	1E/00	1E/01

S		1F	1F	1F/73	1F/53	1F/13	1F/00	1F/13

D		20	20	20/64	20/44	20/04	20/00	20/04

F		21	21	21/66	21/46	21/06	21/00	21/06

G		22	22	22/67	22/47	22/07	22/00	22/07

H		23	23	23/68	23/48	23/08	23/00	23/08

J		24	24	24/6A	24/4A	24/0A	24/00	24/0A

K		25	25	25/6B	25/4B	25/0B	25/00	25/0B

L		26	26	26/6C	26/4C	26/0C	26/00	26/0C

4	;	05	27	05/34	27/3B

5	:	06	27	06/35	27/3A

6	'	07	28	07/36	28/27	07/1E

*	"	37	28	37/2A	28/22	96/00	37/00

CTRL		1D	1D

Z		2C	2C	2C/7A	2C/5A	2C/1A	2C/00	2C/1A

X		2D	2D	2D/78	2D/58	2D/18	2D/00	2D/18

C		2E	2E	2E/63	2E/43	2E/03	2E/00	2E/03

V		2F	2F	2F/76	2F/56	2F/16	2F/00	2F/16

B		30	30	30/62	30/42	30/02	30/00	30/02

N		31	31	31/6E	31/4E	31/0E	31/00	31/0E

M		32	32	32/6D	32/4D	32/0D	32/00	32/0D

ENTER		1C	1C	1C/0D	1C/0D	1C/0A	1C/00	1C/0A

1	<	02	33	02/31	33/3C

2	>	03	34	03/32	34/3E	03/00

3	?	04	35	04/33	35/3F

-	^	4A	07	4A/2D	07/5E	8E/00		07/1E

Shift		2A	2A

ALT		38	38

Char		79	79

<space>		39	39	39/20	39/20	39/20	39/20	39/20

,	,	33	33	33/2C	33/2C		33/00

@	@	03	03	03/40	03/40	03/00	79/00	03/00

MENU		7A	7A	C8/00	C9/00	CA/00	CB/00	CA/00

Shift		36	36

0	CAPS	0B	3A	0B/30

..	SCRL	34	46	34/2E			34/00	BREAK

=	_	0D	0C	0D/3D	0C/5F		83/00	0C/1F

+	%	4E	06	4E/2B	06/25	90/00

SCANCODE TABLE WHEN CHAR FUNCTION IS ON

CHARACTER CODES (hex)

KEY LEGEND	SCANCODE	Normal	Shifted	Control	Alt	Shift+Control

Normal	Shifted	Normal	Shifted	ah/al	ah/al	ah/al	ah/al	ah/al

ESC	PrtScr	01	37	01/1B	PrtScr	01/1B

TAB 		0F 	0F 	0F/09 	0F/00	94/00	A5/00	94/00

F1		3B	3B	DB/00	F4/00	5E/00	68/00	5E/00

F2		3C	3C	DC/00	F5/00	5F/00	69/00	5F/00

F3		3D	3D	DD/00	F6/00	60/00	6A/00	60/00

F4		3E	3E	DE/00	F7/00	61/00	6B/00	61/00

F5		3F	3F	DF/00	F8/00	62/00	6C/00	62/00

F6		40	40	E0/00	F9/00	63/00	6D/00	63/00

F7		41	41	E1/00	FA/00	64/00	6E/00	64/00

F8		42	42	E2/00	FB/00	65/00	6F/00	65/00

F9		43	43	E3/00	FC/00	66/00	70/00	66/00

F10		44	44	E4/00	FD/00	67/00	71/00	67/00

upcurs	PGUP	48	49	48/00	49/00	8D/00		84/00

ON/OFF 	ON/OFF	70	70			A2/00

FILER	SETUP	72	71			AE/00	AB/00	AA/00

COMM	`	73	29	04/EF	29/60	B2/00	AF/00

APPT	~	74	29	05/F9	29/7E	B6/00	B3/00

PHONE	!	75	02	06/AD	02/AD	BA/00	B7/00

MEMO	#	76	04	07/EE	04/23	BE/00	BB/00

123	$	77	05	08/B8	05/24	C2/00	BF/00

+-*/	&	78	08	09/A9	08/26	C6/00	C3/00

(	|	0A	2B	0A/DD	2B/7C		80/00	2B/1C

)	\	0B	2B	0B/29	2B/5C		81/00	2B/1C

bkspc	backspc	0E	0E	0E/08	0E/08	BREAK	0E/00	BREAK

DEL	INS	53	52	53/00	52/00	93/00		92/00

lfcurs	HOME	4B	47	4B/00	47/00	73/00		77/00

dncurs	PGDN	50	51	50/00	51/00	91/00		76/00

rtcurs	END	4D	4F	4D/00	4F/00	74/00		75/00

Q 		10	10	10/A6	10/A6	10/11	10/00	10/11

W 		11	11	11/A7	11/A7	11/17	11/00	11/17

E 		12	12	12/91	12/92	12/05	12/00	12/05

R 		13	13	*****	*****	13/12	13/00	13/12

T 		14	14	*****	*****	14/14	14/00	14/14

Y 		15	15	*****	*****	15/19	15/00	15/19

U		16	16	*****	*****	16/15	16/00	16/15

I		17	17	*****	*****	17/09	17/00	17/09

O		18	18	18/9B	18/9D	18/0F	18/00	18/0F

P		19	19	19/E7	19/E8	19/10	19/00	19/10

7	[\(dd	08	1A	08/AC	1A/5B			1A/1B

8	]\(dd	09	1B	09/AB	1B/5D			1B/1D

9	{	0A	1A	0A/F3	1A/7B			1A/1B

/	}	35	1B	35/F6	1B/7D			1B/1D

***** Mute function enabled.

SCANCODE TABLE WHEN CHAR FUNCTION IS ON

CHARACTER CODES (hex)

KEY LEGEND	SCANCODE	Normal	Shifted	Control	Alt	Shift+Control

Normal	Shifted	Normal	Shifted	ah/al	ah/al	ah/al	ah/al	ah/al

A		1E	1E	1E/86	1E/8F	1E/01	1E/00	1E/01

S		1F	1F	1F/E1	1F/E1	1F/13	1F/00	1F/13

D		20	20	20/D0	20/D1	20/04	20/00	20/04

F		21	21	21/9F	21/9F	21/06	21/00	21/06

G		22	22	22/CF	22/CF	22/07	22/00	22/07

H		23	23	23/BE	23/BE	23/08	23/00	23/08

J		24	24	24/24	24/24	24/0A	24/00	24/0A

K		25	25	25/BD	25/BD	25/0B	25/00	25/0B

L		26	26	26/9C	26/9C	26/0C	26/00	26/0C

4	;	05	27	05/34	27/3B

5	:	06	27	06/35	27/3A

6	'	07	28	07/36	28/27	07/1E

*	"	37	28	37/9E	28/22	96/00	37/00

CTRL		1D	1D

Z		2C	2C	2C/F4	2C/F4	2C/1A	2C/00	2C/1A

X		2D	2D	2D/F8	2D/F8	2D/18	2D/00	2D/18

C		2E	2E	2E/87	2E/80	2E/03	2E/00	2E/03

V		2F	2F	2F/F5	2F/F5	2F/16	2F/00	2F/16

B		30	30	30/FE	30/FE	30/02	30/00	30/02

N		31	31	31/D5	31/D5	31/0E	31/00	31/0E

M		32	32	32/E6	32/E6	32/0D	32/00	32/0D

ENTER		1C	1C	1C/0D	1C/0D	1C/0A	1C/00	1C/0A

1	<	02	33	02/AE	33/FB

2	>	03	34	03/AF	34/FD	03/00

3	?	04	35	04/A8	35/FC

-	^	4A	07	4A/F0	07/5E	8E/00		07/1E

Shift		2A	2A

ALT		38	38

Char		79	79

<space>		39	39	39/20	39/20	39/20	39/20	39/20

,	,	33	33	33/F7	33/F7		33/00

@	@	03	03	03/40	03/40	03/00	79/00	03/00

MENU		7A	7A		C9/00	CA/00	CB/00	CA/00

Shift		36	36

0	CAPS	0B	3A	0B/30

..	SCRL	34	46	34/FA			34/00	BREAK

=	_	0D	0C	0D/F2	0C/5F		83/00	0C/1F

+	%	4E	06	4E/F1	06/25	90/00

MUTE KEY SEQUENCES

CHARACTER CODES (hex)

Normal	Shifted	Control	Alt

KEY SEQUENCE	ah/al	ah/al	ah/al	ah/al

[Char][r][a]	1E/A0 	1E/B5 	1E/01 	1E/00

[Char][r][e]	12/82	12/90 	12/05 	12/00

[Char][r][i]	17/A1	17/D6	17/09	17/00

[Char][r][o]	18/A2 	18/E0 	18/10 	18/00

[Char][r][u]	16/A3	16/E9	16/15	16/00

[Char][r][y]	15/EC  	15/ED	15/19	15/00

[Char][r][n]	31/6E	31/4E	31/0E	31/00

[Char][t][a]	1E/85 	1E/B7 	1E/01 	1E/00

[Char][t][e]	12/8A	12/D4 	12/05 	12/00

[Char][t][i]	17/8D	17/DE	17/09	17/00

[Char][t][o]	18/95 	18/E3 	18/10 	18/00

[Char][t][u]	16/97	16/EB	16/15	16/00

[Char][t][y]	15/79	15/59	15/19	15/00

[Char][t][n]	31/6E	31/4E	31/0E	31/00

[Char][y][a]	1E/83	1E/B6 	1E/01 	1E/00

[Char][y][e]	12/88 	12/D2	12/05 	12/00

[Char][y][i]	17/8C	17/D7	1/09	17/00

[Char][y][o]	18/93	18/E2 	18/10 	18/00

[Char][y][u]	16/96	16/EA	1/15	16/00

[Char][y][y]	15/79	15/59	1/19	15/00

[Char][y][n]	31/6E	31/4E	3/0E	31/00

[Char][u][a]	1E/84 	1E/8E 	1E/01 	1E/00

[Char][u][e]	12/89	12/D3 	12/05 	12/00

[Char][u][i]	17/8B	17/D8	17/09	17/00

[Char][u][o]	18/94 	18/99 	18/10 	18/00

[Char][u][u]	16/81	16/9A	16/15	16/00

[Char][u][y]	15/98	15/59	15/19	15/00

[Char][u][n]	31/6E	31/4E	31/0E	31/00

[Char][i][a]	1E/C6 	1E/C7 	1E/01 	1E/00

[Char][i][e]	12/65	12/45 	12/05 	12/00

[Char][i][i]	17/69	17/49	17/09	17/00

[Char][i][o]	18/E4 	18/E5 	18/10 	18/00

[Char][i][u]	16/75	16/55	16/15	16/00

[Char][i][y]	15/79	15/59	15/19	15/00

[Char][i][n]	31/A4	31/A5	31/0E	31/00

--------------------- end of SCANCODE.DOC -------------------------------------

Everett Kaser                   Hewlett-Packard Company

....hplabs!hp-pcd!everett        work: (503) 750-3569   Corvallis, Oregon

everett%hpcvra@hplabs.hp.com